IDENTIFY

CONSULT

What is the manual task?

Using the body to lift, lower, push, pull, carry or otherwise move, hold or restrain any person, animal or thing.

Is the manual task hazardous?

Application of force:

- Repetitive

- Sustained

- High

- Sudden

Posture:

- Sustained

- Awkward

Movement:

- Repetitive

Exposure to vibration

/

/

/

/

CONSULT

ASSESS

What is the risk of MSD?

- How often and how long are specific postures, movement or forces performed or held?

- What is the duration of the task?

- Does the task involve high or sudden force?

- Does the task involve vibration?

What is the source of risk?

CONTROL

Work area design 
and layout

/

Systems of work

/

Nature, size weight and number of persons, animals or things handled

/

/

CONSULT

Work environment

- Is the task necessary?

- Can the source of risk (work area layout, environment, etc) be changed?

- Can mechanical aids be used to perform the task?

- What training is needed to support the control measures?

/

CONSULT

REVIEW

when the control measure is no longer effective

- before a change at the workplace that is likely to give rise to a new or different health and safety risk that the control measure may not effectively control

- if a new hazard or risk is identified

- if the results of consultation indicate that a review is necessary

- if a health and safety representative at the workplace requests a review.

Work area: fgfgfgfgf

Management representative:

Health and Safety representative and workers taking part:

Date:

Does the task have any of the characteristics of a hazardous manual task? (check  any of the following that apply by double clicking on the box)

| Task | Repetitive or sustained force | High or sudden force | Sustained or awkward postures | Repetitive movement | Exposure to vibration |
| --- | --- | --- | --- | --- | --- |

If you ticked any of the boxes for a particular task, you should do a risk assessment of that task.

A discomfort survey can help identify hazardous manual tasks. Early reporting of symptoms can lead to risk controls being put in place before injury occurs.

The survey sheet below will help you identify and record instances where workers experience discomfort that:

- persists, or

- re-occurs the next day, or

- persists after rostered days off.

Encourage workers to report pain or discomfort at work or at any other time. Follow up the reasons for the problem. Even if only one worker reports problems, assess the presence of a risk factor.

Name (optional):

Date:

Job work location:

Tasks involved:      
Time on this job:	  Less than 3mths		  3mths to 1yr		  1 to 5yrs

Supervisor:

- Do you suffer from swelling, numbness, tingling, ‘pins and needles’ 
stiffness, aches or pains in any part of the body? Indicate in the 
diagrams where the problem occurs.

- Rate the level of discomfort/pain on a scale of 1 to 5

1 		2 		3 		4 		5

Just noticeable	Moderate		Unbearable

- What do you think caused the problem?

Location of task:     					Management rep:

Description of hazardous manual task:     				Health and Safety rep:

Date of assessment:      					Others (workers consultants):

Reason for identification

Existing task				Report of musculoskeletal disorder (MSD)			 Change in task, object or tool

New task					 New information

Step 1 – Does the task involve repetitive or sustained movements, postures or forces?

As a guide:

- repetitive means the movement or force is performed more than twice a minute, and

- sustained means the posture or force is held for more than 30 seconds at a time.

| Postures and Movements (place a tick in the ‘yes’ column each time you observe repetitive movement or sustained posture) |  | Yes  | This action happens when… | because... (describe why) This is the source of the risk | If any boxes are ticked, what are possible controls to reduce the risk? |
| --- | --- | --- | --- | --- | --- |
| BACK |  |  |  |  |  |
| Bending or twisting e.g. more than 20 degrees | Forwards |  |  |  |  |
|  | Sideways |  |  |  |  |
|  | Twisting |  |  |  |  |
| Bending e.g. more than 5 degrees | Backwards |  |  |  |  |
| NECK OR HEAD |  |  |  |  |  |
| Bending or twisting e.g. more than 20 degrees | Forwards |  |  |  |  |
|  | Sideways |  |  |  |  |
|  | Twisting |  |  |  |  |
| Bending e.g. more than 5 degrees | Backwards |  |  |  |  |
| ARMS/HANDS |  |  |  |  |  |
| Working with one or both hands above shoulder height |  |  |  |  |  |
| Reaching forwards or sideways more than 30cm from the body |  |  |  |  |  |
| Reaching behind the body |  |  |  |  |  |
| Excessive bending of the wrist |  |  |  |  |  |
| Twisting, turning, grabbing, picking or wringing actions with the fingers, hands or arms |  |  |  |  |  |
| LEGS |  |  |  |  |  |
| Squatting, kneeling, crawling, lying, semi-lying or jumping |  |  |  |  |  |
| Standing with most of the body’s weight on one leg |  |  |  |  |  |
| VERY FAST MOVEMENTS |  |  |  |  |  |
| Lifting or lowering |  |  |  |  |  |
| Carrying with one hand or one side of the body |  |  |  |  |  |
| Exerting force with one hand or one side of the body |  |  |  |  |  |
| Pushing, pulling or dragging |  |  |  |  |  |
| Very fast actions |  |  |  |  |  |
| Working with the fingers close together or wide apart |  |  |  |  |  |
| Applying uneven, fast or jerky forces |  |  |  |  |  |
| Holding, supporting or restraining anything (including a person, animal or tool) |  |  |  |  |  |

Step 2 – Does the task in step 1 involve long duration?

Tick yes if the task is done for:

| Duration | Yes  | Comments |
| --- | --- | --- |
| More than 2 hours over a whole shift |  |  |
| Continually for more than 30 minutes at a time |  |  |
| IF YOU TICKED YES THEN THE TASK IS A RISK AND MUST BE CONTROLLED |  |  |

Step 3 – Does the task involve high or sudden force?

| FORCES (Tick yes if the task involves any of the following high or sudden forces, even if the force is applied only once) | Yes  | This action happens when… | because... (describe why) This is the source of the risk | If any boxes are ticked, what are possible controls to reduce the risk? |
| --- | --- | --- | --- | --- |
| Lifting, lowering or carrying heavy loads |  |  |  |  |
| Throwing or catching |  |  |  |  |
| Hitting, kicking or jumping |  |  |  |  |
| Applying a sudden or unexpected force including: - handling a live person or animal - applying uneven, fast or jerky forces during lifting, carrying, pushing or pulling, or - pushing or pulling objects that are hard to move or stop e.g. a trolley |  |  |  |  |
| Exerting force while in a bent, twisted or awkward posture including: - supporting items with hands are above shoulder height - moving items when legs are in an awkward posture, working with fingers pinched together or held wide apart, or - using a finger grip, pinch grip or an open handed grip |  |  |  |  |
| Exerting a force with the non-preferred hand |  |  |  |  |
| Needing to use two hands to operate a tool designed for one hand |  |  |  |  |
| The task can only be done for short periods of time |  |  |  |  |
| Two or more people need to be assigned to handle a heavy, awkward or bulky load |  |  |  |  |
| Workers report pain or significant discomfort during or after the task |  |  |  |  |
| Stronger workers assigned to do the task |  |  |  |  |
| Employees say the task is physically very strenuous or difficult to do |  |  |  |  |
| Workers think the task should be done by more than one person, or seek help to do the task as it requires high force |  |  |  |  |

Step 4 – Is there hand, arm or whole body vibration?

Tick yes if any of the following environmental factors are present in the task.

| Environmental factors | YES |
| --- | --- |
| Driving for long periods |  |
| Driving on rough roads |  |
| Frequent use of hand powered tools or use for long periods |  |
| Using high grip forces or awkward postures when using power tools |  |
| Use of machines or tools where the manufacturer’s handbook warns of vibration |  |
| Workers being jolted or continuously shaken |  |
| Use of a vehicle or tool not suitable for the environment or task |  |

Step 5 – Is there a risk?

Did you answer yes in step 1 and step 2? The task is a risk. Risk control is required.

Did you answer yes in step 3? The task is a risk. Risk control is required.

Did you answer yes in step 4? This task requires further investigation

To aid prioritisation of timing and resourcing risk controls you may also need to consider:

- Number of ticks or risk factors.

- Additional factors such as injuries associated with the task.

These items capture degree and likelihood of harm. You will also need to consider the availability and suitability of risk controls for the task.

## RISK CONTROL

What needs to be fixed to control the risk?

You may need to use a combination of risk controls to eliminate or minimise the risk as far as reasonably practicable.

Can you stop doing the task or part of the task?

How?

Yes

No

Can you eliminate or reduce the risk by doing one or more of these things?

- altering the work area design and layout

- altering the equipment, machinery and loads handled

- altering the workplace environment

- changing the work organisation, work practices or systems of work

Yes

How?

What information, instruction, training and supervision is necessary to make the new procedure work properly?

Can you reduce the risk with information, instruction, training and supervision? How?

No

| Type of plant | MSD risk | Possible design solution |
| --- | --- | --- |
| Road-making machinery | Repetitive or sustained twisting of the neck and body while reversing. This is caused by the seat being fixed in a forward-facing position. | Design a swivel seat-mount together with two sets of controls, or controls that move with seat rotation. |
| Forklifts | Sustained exposure to whole-body vibration transferred through the seat. Repetitive or sustained bending of the neck and back to see the work properly (for example, continually looking up to place loads on high shelves). | Install damping mechanisms in the seat, cabin and vehicle suspension. Install visual aids such as mirrors or a video camera and screen. |
| Wrapping machines on process lines | Strain on the lower back when handling heavy rolls of plastic wrapping in awkward and twisted postures, often above shoulder height. This is caused by inappropriate design and positioning of the roll spindle and by restricted access. | Design the spindle to be adjustable. This allows the rolls to be loaded at a suitable height and orientation, and eliminates the need to lift them. Design equipment to help worker load rolls. Locate the spindle in an accessible placeon the plant. Provide information about how to install the plant in a way that allows adequate access. |
| Power drills | Prolonged use of the forearm muscles and wrist caused by a heavy or poorly balanced drill. Exposure to vibration or impact shock recoil from hammer drills. Excessive force needed to grip and control the tool to counter the effect of vibration and impact shocks. | Design drills to be as light as possible. Design drills with the handle under the drill’s centre of gravity. Design plant to reduce shock and vibration. Provide a suitable way of holding the tool with both hands. |
| Pliers | Pressure to the palm of the hand caused by handles that are too short. Prolonged use of the forearm muscles and compression of the wrist caused by using pliers with straight handles. | Design pliers with handles that extend beyond the palm. Design pliers with bent handles so that the user can maintain a straight wrist. |
| Crimping, clamping and cutting tools | Excessive force with outstretched fingers required to grip handles that are too wide apart. | Design handles with a grip span of 10 cm or less. |
| Chainsaws | Excessive vibration. High force required to handle the chainsaw. | Design to reduce vibration. Design the chainsaw to be as light as possible, and provide well-placed handles. |
| Chairs | Poorly designed chairs that cannot be adjusted provide little back support and cause workers to adopt poor postures and movements. | Follow existing design guidelines for chairs and consider how the chair will be used in the workplace. |
| Work-benches, workstations and other work surfaces | Workstations that cannot be adjusted result in unnecessary reaching, bending and exertion of force. | Design workstations to be adjustable. Alternatively, dimensions should suit as many workers as possible. |

| Method | More information |
| --- | --- |
| University of Michigan 3 Dimensional Static Strength Prediction Program Computer Software OR University of Michigan 2 Dimensional Static Strength Prediction Computer Software | www.umich.edu |
| 1991 NIOSH Lifting Equation (National Institute of Occupational Safety and Health – USA) | USA National Institute of Occupational Safety and Health http://www.cdc.gov/niosh/docs/94-110/pdfs/94-110.pdf http://www.cdc.gov/niosh/docs/94-110/pdfs/94-110.pdf |
| Snook and Ciriello’s Tables of Maximum Acceptable Weights and Forces | Snook and Ciriello, “The design of manual handling tasks: revised tables of maximum acceptable weights and forces”, in Ergonomics, 1991, vol. 34, no. 9, pp. 1197-1213. |
| Rapid Entire Body Assessment (REBA) | Hignett, S and McAtamney, L., Applied Ergonomics, 2000, vol. 31, pp. 201-205 |
| Posture, Activity, Tools, Handling (PATH) | PATH : A work sampling-based approach to ergonomic job analysis for construction and other non-repetitive work. Applied Ergonomics 1996, vol. 27, no. 3, pp. 177-187 |
| Manual Tasks Risk Assessment Tool (ManTRA) V2.0 | Burgess-Limerick, L., Straker, L., Pollock, C., Egeskov, R. 2004. Manual Risk Assessment Tool (ManTRA) V2.0. School of Human Movement Studies, The University of Queensland, Australia. http://ergonomics.uq.edu.au/download/mantra2.pdf |
| PErforM | Burgess-Limerick, R. (2004): PErforM and ManTRA. Ergonomics Australia, 18(4), 10-13. |
| OCRA Index and Checklist | Colombini, D., Occhipinti, E., & Grieco, A. (2002). Risk assessment and management of repetitive movements and exertions of upper limbs : job analysis, OCRA risk indices, prevention strategies and design principles. Oxford: Elsevier. |
| The Work Organisation Assessment Questionnaire (WOAQ) | EEF (2004). Work Organisation Assessment Questionnaire: A tool for the risk management of stress. London: EEF. |
| MSD risk assessment questionnaire | Wendy Macdonald, Owen Evans and Ross Armstrong, 2007, Centre for Ergonomics & Human Factors, Faculty of Health Sciences, La Trobe University. Research on the Prevention of Work-Related Musculoskeletal Disorders – Stage 2 A study of a small sample of workplaces in high risk industries. Department of Employment and Workplace Relations. OHS EXPERT RESEARCH PANEL. Project No. CIR 1001439-07 |